Caliper Measurement Work Instruction

Product Alpha critical-width verification for ForgeLine

| Document ID | Effective date | Product | Tolerance |
| --- | --- | --- | --- |
| FL-QWI-018 | 2026-08-11 | Product Alpha | ±0.18 mm |

# Purpose

Use this controlled instruction to verify Product Alpha critical width after the 2026-08-11 process update. The acceptance band is nominal value ±0.18 mm.

# Preconditions

Use a clean, damage-free caliper and allow the part to stabilize at the inspection bench. Do not force the jaws or measure across burrs.

# Disposition

Record each reading. If any reading is outside the acceptance band, stop the lot, identify the sample, and follow NCR-026 containment.

# Controlled steps

| Step | Operator action | Acceptance evidence |
| --- | --- | --- |
| 1 | Clean jaws and the Product Alpha measurement surface. | No debris or visible burr at contact point. |
| 2 | Close jaws gently and verify zero before each sample set. | Display indicates zero; log the check. |
| 3 | Measure the specified critical width at inspection point IP-03. | Reading is within nominal ±0.18 mm. |
| 4 | Record result and release or contain the sample. | Traceable inspection record is complete. |