Gage Zeroing Instruction

Pre-use confirmation for dimensional inspection devices

| Document ID | Effective date | Product | Tolerance |
| --- | --- | --- | --- |
| FL-QWI-018 | 2026-08-11 | Product Alpha | ±0.18 mm |

# Purpose

Establish a repeatable zeroing routine before Product Alpha or Product Beta dimensional checks.

# Zeroing method

Wipe the contact faces with a lint-free synthetic wipe. Close slowly until both faces touch, then set the display to zero. Repeat once to confirm stability.

# Escalation

If the display cannot hold zero or the motion is rough, label the device unavailable and notify the quality lead before inspection begins.

# Controlled steps

| Step | Operator action | Acceptance evidence |
| --- | --- | --- |
| 1 | Inspect gage condition and clean contact faces. | No damage, residue, or loose parts. |
| 2 | Close faces without force and set zero. | Zero displayed. |
| 3 | Open and close a second time. | Repeat zero holds. |
| 4 | Record status in shift-start check log. | Device status is traceable. |